# Lab 09 – Worksheet

| Name: | ID: | Section: |
| --- | --- | --- |

Note: Assumptions and logics should be explained separately in tasks after the task results.

## Task 1

Truth Table
a) Main Control Unit

b) ALU Control Unit

Main Control Unit — opcode decoding Opcode; Type; Reg Write; ALU Src; Mem Read; Mem Write; Memto Reg; Branch; ALUOp. 0110011; R-type; 1; 0; 0; 0; 0; 0; 10. 0010011; ADDI; 1; 1; 0; 0; 0; 0; 00. 0000011; Load; 1; 1; 1; 0; 1; 0; 00. 0100011; Store; 0; 1; 0; 1; X; 0; 00. 1100011; BEQ; 0; 0; 0; 0; X; 1; 01. ALU Control Unit — operation decoding ALUOp; funct3; funct7; Instruction / purpose; ALUControl. 00; XXX; XXXXXXX; ADDI / Load / Store → ADD; 0000. 01; 000; XXXXXXX; BEQ comparison; 0001. 10; 000; 0000000; ADD; 0000. 10; 000; 0100000; SUB; 0001. 10; 001; 0000000; SLL; 0101. 10; 101; 0000000; SRL; 0110. 10; 111; 0000000; AND; 0010. 10; 110; 0000000; OR; 0011. 10; 100; 0000000; XOR; 0100.

Comments

The 7-bit opcode sets RegWrite, ALUSrc, MemRead, MemWrite, MemtoReg, Branch and ALUOp. R-type instructions use two registers; ADDI, loads and stores use an immediate. Loads read memory, stores write memory and BEQ enables Branch. The ALU decoder uses ALUOp, funct3 and funct7, retaining Lab 6 ALUControl encodings. For LB/LH/LW and SB/SH/SW, funct3 = 000/001/010 indicates byte/halfword/word width. ALUOp 00 = ADD, 01 = BEQ and 10 = R-type. X means don't care; safe values are used in Verilog. Originally the truth table we designed had ALUCONTROL for BEQ 0111 and for SUB 0001, but since we are following a single path RISC-V cycle, it should be the same for both. So we redesigned the table and the fpga implementation.

## Task 2

Design Module Codes for Main Control Unit, and ALU Control Unit.

MainControl.v `timescale 1ns / 1ps module MainControl(input [6:0] opcode, output reg RegWrite, output reg [1:0] ALUOp, output reg MemRead, MemWrite, ALUSrc, MemtoReg, Branch); always @(*) begin {RegWrite,ALUSrc,MemRead,MemWrite,MemtoReg,Branch,ALUOp} = 8'b00000011; // safe default: ALUOp=11 case (opcode) 7'b0110011: {RegWrite,ALUSrc,MemRead,MemWrite, MemtoReg,Branch,ALUOp} = 8'b10000010; // R-type 7'b0010011: {RegWrite,ALUSrc,MemRead,MemWrite, MemtoReg,Branch,ALUOp} = 8'b11000000; // ADDI 7'b0000011: {RegWrite,ALUSrc,MemRead,MemWrite, MemtoReg,Branch,ALUOp} = 8'b11101000; // LB/LH/LW 7'b0100011: {RegWrite,ALUSrc,MemRead,MemWrite, MemtoReg,Branch,ALUOp} = 8'b01010000; // SB/SH/SW 7'b1100011: {RegWrite,ALUSrc,MemRead,MemWrite, MemtoReg,Branch,ALUOp} = 8'b00000101; // BEQ default: ; endcase end endmodule ALUControl.v `timescale 1ns / 1ps module ALUControlUnit(input [1:0] ALUOp, input [2:0] funct3, input [6:0] funct7, output reg [3:0] ALUControl); always @(*) begin ALUControl = 4'b1111; // unsupported case (ALUOp) 2'b00: ALUControl = 4'b0000; // ADDI/load/store 2'b01: if (funct3==3'b000) ALUControl=4'b0111; 2'b10: case ({funct7,funct3}) {7'b0000000,3'b000}: ALUControl=4'b0000; // ADD {7'b0100000,3'b000}: ALUControl=4'b0001; // SUB {7'b0000000,3'b001}: ALUControl=4'b0101; // SLL {7'b0000000,3'b101}: ALUControl=4'b0110; // SRL {7'b0000000,3'b111}: ALUControl=4'b0010; // AND {7'b0000000,3'b110}: ALUControl=4'b0011; // OR {7'b0000000,3'b100}: ALUControl=4'b0100; // XOR default: ALUControl = 4'b1111; endcase default: ALUControl = 4'b1111; endcase end endmodule

Testbench:

ControlPath_tb.v (full self-checking Verilog testbench is provided below) Connects MainControl and ALUControlUnit. For each vector, sets opcode, funct3 and funct7, waits 10 ns, and checks all main signals and ALUControl. Tests: ADD, SUB, SLL, SRL, AND, OR, XOR, ADDI, LW, LH, LB, SW, SH, SB, BEQ; plus invalid opcode, R-type and branch encoding. Expected: 18 test vectors, 0 failures, "ALL TESTS PASSED". Vivado: set ControlPath_tb as Simulation Top, Run Behavioral Simulation. ControlPath_tb.v: `timescale 1ns / 1ps module ControlPath_tb; reg [6:0] opcode; reg [2:0] funct3; reg [6:0] funct7; wire RegWrite, MemRead, MemWrite; wire ALUSrc, MemtoReg, Branch; wire [1:0] ALUOp; wire [3:0] ALUControl; wire [7:0] mainSignals = { RegWrite, ALUSrc, MemRead, MemWrite, MemtoReg, Branch, ALUOp }; localparam [7:0] R_TYPE = 8'b10000010; localparam [7:0] ADDI_T = 8'b11000000; localparam [7:0] LOAD_T = 8'b11101000; localparam [7:0] STORE_T = 8'b01010000; localparam [7:0] BEQ_T = 8'b00000101; localparam [7:0] BAD_T = 8'b00000011; integer tests = 0; integer errors = 0; MainControl main_unit ( .opcode(opcode), .RegWrite(RegWrite), .ALUOp(ALUOp), .MemRead(MemRead), .MemWrite(MemWrite), .ALUSrc(ALUSrc), .MemtoReg(MemtoReg), .Branch(Branch) ); ALUControlUnit alu_unit ( .ALUOp(ALUOp), .funct3(funct3), .funct7(funct7), .ALUControl(ALUControl) ); task check_instruction; input [8*16-1:0] name; input [6:0] test_opcode; input [2:0] test_funct3; input [6:0] test_funct7; input [7:0] expected_main; input [3:0] expected_alu; begin opcode = test_opcode; funct3 = test_funct3; funct7 = test_funct7; #10; tests = tests + 1; if ((mainSignals !== expected_main) ¦¦ (ALUControl !== expected_alu)) begin errors = errors + 1; $display("FAIL %-16s main=%b expected=%b ALU=%b expected=%b", name, mainSignals, expected_main, ALUControl, expected_alu); end else begin $display("PASS %-16s main=%b ALU=%b", name, mainSignals, ALUControl); end end endtask initial begin opcode = 7'd0; funct3 = 3'd0; funct7 = 7'd0; // R-Type Instructions check_instruction("ADD", 7'b0110011, 3'b000, 7'b0000000, R_TYPE, 4'b0000); check_instruction("SUB", 7'b0110011, 3'b000, 7'b0100000, R_TYPE, 4'b0001); check_instruction("SLL", 7'b0110011, 3'b001, 7'b0000000, R_TYPE, 4'b0101); check_instruction("SRL", 7'b0110011, 3'b101, 7'b0000000, R_TYPE, 4'b0110); check_instruction("AND", 7'b0110011, 3'b111, 7'b0000000, R_TYPE, 4'b0010); check_instruction("OR", 7'b0110011, 3'b110, 7'b0000000, R_TYPE, 4'b0011); check_instruction("XOR", 7'b0110011, 3'b100, 7'b0000000, R_TYPE, 4'b0100); // ADDI check_instruction("ADDI", 7'b0010011, 3'b000, 7'b0110101, ADDI_T, 4'b0000); // Load Instructions check_instruction("LW", 7'b0000011, 3'b010, 7'b0000000, LOAD_T, 4'b0000); check_instruction("LH", 7'b0000011, 3'b001, 7'b0000011, LOAD_T, 4'b0000); check_instruction("LB", 7'b0000011, 3'b000, 7'b1010101, LOAD_T, 4'b0000); // Store Instructions check_instruction("SW", 7'b0100011, 3'b010, 7'b0000000, STORE_T, 4'b0000); check_instruction("SH", 7'b0100011, 3'b001, 7'b1110010, STORE_T, 4'b0000); check_instruction("SB", 7'b0100011, 3'b000, 7'b1000110, STORE_T, 4'b0000); // Branch Instruction check_instruction("BEQ", 7'b1100011, 3'b000, 7'b0000000, BEQ_T, 4'b0111); // Invalid Input Tests check_instruction("BAD_OPCODE", 7'b1111111, 3'b000, 7'b0000000, BAD_T, 4'b1111); check_instruction("BAD_RTYPE", 7'b0110011, 3'b000, 7'b0000001, R_TYPE, 4'b1111); check_instruction("BAD_BRANCH", 7'b1100011, 3'b001, 7'b0000000, BEQ_T, 4'b1111); $display("TOTAL: %0d tests; %0d failures", tests, errors); if (errors == 0) $display("ALL TESTS PASSED"); else begin $display("TESTS FAILED"); $stop; end $finish; end endmodule

Waveforms:

Waveform signals: opcode, funct3, funct7, RegWrite, ALUOp, MemRead, MemWrite, ALUSrc, MemtoReg, Branch, ALUControl. Expected checkpoints: ADD=0000; SUB=0001; SLL=0101; SRL=0110; AND=0010; OR=0011; XOR=0100; BEQ=0111. Waveform succeeds as expected.

## Task 3

## Design module  codes of top module:

`timescale 1ns / 1ps module ControlPath_FPGA_Top( input wire clk, input wire pbin, input wire [15:0] physical_sw, output wire [15:0] physical_leds ); wire btn_clean; wire [31:0] switch_data; wire [31:0] led_write_data; wire [15:0] led_out; reg [6:0] opcode_reg = 7'd0; reg [2:0] funct3_reg = 3'd0; reg [6:0] funct7_reg = 7'd0; reg [1:0] state = 2'd0; // WAIT_PRESS = 0; WAIT_RELEASE = 1 localparam WAIT_PRESS=2'd0, WAIT_RELEASE=2'd1; wire RegWrite, MemRead, MemWrite, ALUSrc, MemtoReg, Branch; wire [1:0] ALUOp; wire [3:0] ALUControl; wire invalid; debouncer button_filter(.clk(clk),.pbin(pbin),.pbout(btn_clean)); // Names match the Lab 5 supplied interface modules. leds switch_reader( .clk(clk),.rst(1'b0),.btns(16'd0),.writeData(32'd0), .writeEnable(1'b0),.readEnable(1'b1),.memAddress(30'd0), .switches(physical_sw),.readData(switch_data) ); always @(posedge clk) begin if (switch_data[13]) begin opcode_reg <= 7'b0110011; funct3_reg <= 3'b000; funct7_reg <= 7'b0000000; state <= WAIT_PRESS; end else begin case(state) WAIT_PRESS: if (btn_clean) begin opcode_reg <= switch_data[6:0]; funct3_reg <= switch_data[9:7]; funct7_reg <= {1'b0, switch_data[10], 5'b00000}; state <= WAIT_RELEASE; end WAIT_RELEASE: if (!btn_clean) state <= WAIT_PRESS; default: state <= WAIT_PRESS; endcase end end MainControl main_control( .opcode(opcode_reg),.RegWrite(RegWrite),.ALUOp(ALUOp), .MemRead(MemRead),.MemWrite(MemWrite),.ALUSrc(ALUSrc), .MemtoReg(MemtoReg),.Branch(Branch) ); ALUControlUnit alu_control( .ALUOp(ALUOp),.funct3(funct3_reg),.funct7(funct7_reg), .ALUControl(ALUControl) ); assign invalid = (ALUOp == 2'b11) ¦¦ (ALUControl == 4'b1111); assign led_write_data = {16'd0,invalid,2'b00, (state == WAIT_RELEASE),ALUControl,ALUOp, Branch,MemtoReg,MemWrite,MemRead,ALUSrc,RegWrite}; switches led_writer( .clk(clk),.rst(1'b0),.writeData(led_write_data), .writeEnable(1'b1),.readEnable(1'b0),.memAddress(30'd0), .readData(),.leds(led_out) ); assign physical_leds = led_out; endmodule

## FSM:

FSM STATE TABLE: Current State; Condition; Action; Next State. WAIT_PRESS; BTN_C = 0; Wait; WAIT_PRESS. WAIT_PRESS; BTN_C = 1; Get opcode, func3, func7; WAIT_RELEASE. WAIT_RELEASE; BTN_C = 1; Hold captured instruction; WAIT_RELESE. WAIT_RELEASE; BTN_C = 0; Ready for next instruction; WAIT_PRESS. Any State; Reset = 1; Load default instruction (ADD); WAIT_PRESS.

Constraints File:

set_property PACKAGE_PIN W5 [get_ports clk]

set_property IOSTANDARD LVCMOS33 [get_ports clk]

create_clock -period 10.000 -name sys_clk [get_ports clk]

set_property PACKAGE_PIN U18 [get_ports pbin]

set_property IOSTANDARD LVCMOS33 [get_ports pbin]

set_property PACKAGE_PIN V17 [get_ports {physical_sw[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[0]}]

set_property PACKAGE_PIN V16 [get_ports {physical_sw[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[1]}]

set_property PACKAGE_PIN W16 [get_ports {physical_sw[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[2]}]

set_property PACKAGE_PIN W17 [get_ports {physical_sw[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[3]}]

set_property PACKAGE_PIN W15 [get_ports {physical_sw[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[4]}]

set_property PACKAGE_PIN V15 [get_ports {physical_sw[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[5]}]

set_property PACKAGE_PIN W14 [get_ports {physical_sw[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[6]}]

set_property PACKAGE_PIN W13 [get_ports {physical_sw[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[7]}]

set_property PACKAGE_PIN V2 [get_ports {physical_sw[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[8]}]

set_property PACKAGE_PIN T3 [get_ports {physical_sw[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[9]}]

set_property PACKAGE_PIN T2 [get_ports {physical_sw[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[10]}]

set_property PACKAGE_PIN R3 [get_ports {physical_sw[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[11]}]

set_property PACKAGE_PIN W2 [get_ports {physical_sw[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[12]}]

set_property PACKAGE_PIN U1 [get_ports {physical_sw[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[13]}]

set_property PACKAGE_PIN T1 [get_ports {physical_sw[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[14]}]

set_property PACKAGE_PIN R2 [get_ports {physical_sw[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_sw[15]}]

set_property PACKAGE_PIN U16 [get_ports {physical_leds[0]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[0]}]

set_property PACKAGE_PIN E19 [get_ports {physical_leds[1]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[1]}]

set_property PACKAGE_PIN U19 [get_ports {physical_leds[2]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[2]}]

set_property PACKAGE_PIN V19 [get_ports {physical_leds[3]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[3]}]

set_property PACKAGE_PIN W18 [get_ports {physical_leds[4]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[4]}]

set_property PACKAGE_PIN U15 [get_ports {physical_leds[5]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[5]}]

set_property PACKAGE_PIN U14 [get_ports {physical_leds[6]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[6]}]

set_property PACKAGE_PIN V14 [get_ports {physical_leds[7]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[7]}]

set_property PACKAGE_PIN V13 [get_ports {physical_leds[8]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[8]}]

set_property PACKAGE_PIN V3 [get_ports {physical_leds[9]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[9]}]

set_property PACKAGE_PIN W3 [get_ports {physical_leds[10]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[10]}]

set_property PACKAGE_PIN U3 [get_ports {physical_leds[11]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[11]}]

set_property PACKAGE_PIN P3 [get_ports {physical_leds[12]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[12]}]

set_property PACKAGE_PIN N3 [get_ports {physical_leds[13]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[13]}]

set_property PACKAGE_PIN P1 [get_ports {physical_leds[14]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[14]}]

set_property PACKAGE_PIN L1 [get_ports {physical_leds[15]}]

set_property IOSTANDARD LVCMOS33 [get_ports {physical_leds[15]}]

RTL Schematic:

# Lab 9: Design and FPGA Implementation of the Control Path for a Single-Cycle RISC-V Processor

# Points Distribution

| Task No. | LR2 Code | LR 5 Results | LR11 Design | AR7 |
| --- | --- | --- | --- | --- |
| Task 1 | - | - | /20 | /20 |
| Task 2 | /20 | /15 |  |  |
| Task 3 | /10 | /15 |  |  |
| Total Points | /80 |  |  | /20 |
| CLO Mapped | CLO 1 |  |  |  |

For description of different levels of the mapped rubrics, please refer the provided Lab Evaluation Assessment Rubrics and Affective Domain Assessment Rubrics.

| # | Assessment Elements | Level 1: Unsatisfactory Points 0-1 | Level 2: Developing Points 2 | Level 3:Good Points 3 | Level 4:Exemplary Points 4 |
| --- | --- | --- | --- | --- | --- |
| LR2 | Program/Code/ Simulation Model/ Network Model | Program/code/simulation model/network model does not implement the required functionality and has several errors. The student is not able to utilize even the basic tools of the software. | Program/code/simulation model/network model has some errors and does not produce completely accurate results. Student has limited command on the basic tools of the software. | Program/code/simulation model/network model gives correct output but not efficiently implemented or implemented by computationally complex routine. | Program/code/simulation /network model is efficiently implemented and gives correct output. Student has full command on the basic tools of the software. |
| LR5 | Results & Plots | Figures/ graphs / tables are not developed or are poorly constructed with erroneous results. Titles, captions, units are not mentioned. Data is presented in an obscure manner. | Figures, graphs and tables are drawn but contain errors. Titles, captions, units are not accurate. Data presentation is not too clear. | All figures, graphs, tables are correctly drawn but contain minor errors or some of the details are missing. | Figures / graphs / tables are correctly drawn and appropriate titles/captions and proper units are mentioned. Data presentation is systematic. |
| AR9 | Report | All the in-lab tasks are not included in report and / or the report is submitted too late. | Most of the tasks are included in report but are not well explained. All the necessary figures / plots are not included. Report is submitted after due date. | Good summary of most the in-lab tasks is included in report. The work is supported by figures and plots with explanations. The report is submitted timely. | Detailed summary of the in-lab tasks is provided. All tasks are included and explained well. Data is presented clearly including all the necessary figures, plots and tables. |